Initial data stream for

Philips PCF2119X display

(C307 MID, HIGH)

I2C

| Data byte BIN HEX |  | Instruction name |
| --- | --- | --- |
| I2C Start Cond. | --- | See I2C specification for details |
| 01110110 | 76 | Slave address for write |
| 00000000 | 00 | Controll Byte for following controll Bytes |
| 00111101 | 3D | Function set, extended command set |
| 10101000 | BD | Set VLCD* |
| 01000000 | 40 | Reserved (stabilizing contrast) |
| 00010000 | 10 | Set temperature coefficient |
| 00000111 | 07 | Display configuration |
| 00001000 | 08 | Icon mode (disable all) |
| 00000011 | 03 | Set screen config |
| 00111100 | 3C | Function set, basic command set |
| 00000110 | 06 | Entry mode set |
| 00010100 | 14 | Cursor, display shift |
| 00001110 | 0E | Display controll |
| 00000001 | 01 | Clear display |
| 00000010 | 02 | Return home |
| 10000010 | 82 | Set DDRAM adress** |
| I2C Stop Cond. | --- | See I2C specification for details |

*Supported VLCD values: 32…63 (0x20…0x3f) VLCD value is writted in 6Lsb (VLCD command format is: 10VVVVVV, where VVVVVV is the vlcd value)

**DDRAM adresses:	Row1:  0x02…..0x0F

Row2:	0x40…...0x4F

DDRAM Address value is writted in 7Lsb…

Write Data to display

| I2C Start Cond. | --- | See I2C specification for details |
| --- | --- | --- |
| 01110110 | 76 | Slave address for write |
| 01000000 | 40 | Controll Byte for write data |
| xxxxxxxx | XX | First Data byte |
| . | . | . |
| . | . | n. Data byte |
| . | . | . |
| xxxxxxxx | XX | Last Data byte |
| I2C Stop Cond. | --- | See I2C specification for details |